Supplement: Manufacturing Instructions for CLOS-IT

Materials:

1 18x18cm plastic frame & mold		1 	measuring cup

1 pack 	Powermesh			1 	mixing stick

1 tub 	Dragon Skin 30 			1 set	Silc Pig dyes

1 tub	Ecoflex 00-30			1 tube 	silicone adhesive (SilBond)

1 tub	Soma Foama

Instructions:		*Fascia and skin are more efficiently made two-at-a-time. If doing this, double the quantities listed below.

1 Preparing Fascia

- Secure 3 layers of Powermesh in the 18x18cm plastic frame. Adjust as needed so the fabric lies flat against the benchtop.

- Combine 35cc of each Dragon Skin 30 component (70cc total) and stir thoroughly.

- Pour the mixture onto the layered Powermesh and spread evenly with the mixing stick.

Fascia layer prior to curing

Let cure for 16h.

2 Adding Musculature

- Combine 140 cc of each Ecoflex 00-30 component with a small dab of the blood-colored dye and stir.

- Place the mold for the abdominal muscles onto the cured fascia layer, and evenly pour the Ecoflex into the four spaces created by the mold.

Muscle layer prior to curing

Let cure for 4h.

3 Adding Subcutaneous Fat

- Remove the plastic frame from the combined fascia and muscle clayers and place the frame flat on the benchtop.

- Combine 70cc of each Soma Foama component with a small dab of yellow dye and stir quickly as the mixture will begin to foam somewhat rapidly.

- Pour the mixture into the plastic frame in three even columns. Because the mixture tends to cure into an uneven surface if poured irregularly or brushed, it is important to pour as evenly as possible.

Fat layer prior to curing

After 15min, peel the foam away from the benchtop. Cure time is 2h, but if left for that long, it tends to adhere very tightly to the benchtop. At 15min though, it peels away easily.

- Apply an even layer of silicone adhesive to the fascia, on the side opposite the muscle layer, and lay the Soma Foama over it. Press the two layers together so they cure tightly together. Let cure.

4 Skin

- Secure 1 layer of Powermesh in the 18x18cm plastic frame. Adjust as needed so the fabric lies flat.

- Combine 35cc of each Ecoflex 00-30 component with a small dab of flesh-colored dye and stir well.

- Pour the mixture onto the Powermesh and spread evenly. Let cure for 4h.

Skin layer post-cure

Peel the cured product from the benchtop and affix to the fat layer using the silicone adhesive.

Fat layer with silicone glue

Once cured, your laparotomy trainer is ready for use.